Comparing a Learned Policy and a Model-Based IK/TROT Controller for Quadruped Locomotion in Isaac Sim

A. V. Papin

Bauman Moscow State Technical University

Moscow, Russia

papinav@student.bmstu.ru

Abstract

Quadruped robots are increasingly used in inspection, logistics, and

search-and-rescue, and their development relies heavily on physics

simulation. We compare two different locomotion-control

paradigms for the Unitree Go2 quadruped: a pre-trained

reinforcement-learning (RL) policy distributed with NVIDIA Isaac Sim,

and a classical model-based controller that implements a TROT gait

generator with analytic inverse kinematics (IK), written in Rust and

integrated through ROS 2. Both controllers drive the robot through the same

low-level joint-position interface, emulating the robot's native

low-level mode. Experiments in Isaac Sim 6.0 with IsaacLab 3.0, with

three runs per controller at a commanded speed of 0.3 m/s, show that both

paradigms walk at comparable speed (model-based 0.215 m/s, RL 0.231 m/s)

and comparable cost of transport (2.83 vs 2.84). The RL policy is markedly

more stable (maximum roll 2.3° vs 27.8°, lateral drift 0.37 m vs 1.09 m)

and tracks the commanded velocity over the full range 0.1–0.4 m/s, whereas

the model-based controller is stable only near 0.3 m/s. Conversely, the

model-based controller holds body height more tightly (standard deviation

0.025 m vs 0.040 m) and is fully deterministic, requiring no GPU training

and no learned weights. We also report five concrete integration defects

found and fixed in the model-based controller, and show that it was

unpredictably coupled to real time, while the RL policy is not. The

model-based controller is a transparent baseline and a reliable fallback.

Keywords — quadruped robot, Isaac Sim, inverse kinematics, TROT gait,

reinforcement learning, locomotion control (key words)

Introduction

Legged robots are attractive where wheeled platforms are ineffective:

inspection of unstructured environments, search and rescue, logistics, and

operation on rough terrain. Among commercially available quadrupeds, the

Unitree Go2 [1] is widely used in research because of its open low-level

control interface, its SDK, and the availability of simulation assets.

Developing and validating locomotion controllers requires physics

simulation; NVIDIA Isaac Sim and IsaacLab [2], [3] have become a de-facto

standard for GPU-accelerated training and testing of legged robots.

Two fundamentally different paradigms are used to control walking. The

first is learned policies: a neural network maps observations to joint

commands [4]–[7]. Such policies are flexible and can handle complex

terrain, but they require GPU training, behave as a black box, generalize

poorly outside the training distribution, and do not always recover after a

fall. The second is model-based control: deterministic algorithms that

combine a gait generator with inverse kinematics [8]–[11], often with

an inertial attitude-compensation loop. Model-based controllers are

transparent and can run on a CPU, but they are less adaptive to unexpected

disturbances.

Direct comparisons of the two paradigms in a single simulator, on a single

robot, and through a single interface remain rare. The closest work [12]

compares model-predictive control (MPC) and RL in MuJoCo on the Unitree

Go1. Our work differs in three ways: the model-based controller is an

analytic IK/TROT controller (not MPC), the environment is Isaac Sim /

IsaacLab (not MuJoCo), and the robot is the Unitree Go2 with the official

NVIDIA RL policy. Other work trains RL policies for the Go1 with domain

randomization and transfers them to hardware [13]; the comparison there is

against the built-in controller. Kine2Go [14] provides a kinematic dataset

for the Go2 and confirms the popularity of the platform, but does not

compare against a model-based controller.

The contributions of this paper are: (1) the implementation of a

model-based IK/TROT controller in Rust and its integration with Isaac Sim

through IsaacLab and ROS 2; (2) a direct comparison against the official

NVIDIA RL policy on a single asset and through one low-level joint

interface; (3) a set of practical integration patterns (joint re-mapping,

angle convention calibration, amplitude limiting, and time-base

correction); (4) a quantitative evaluation of stability, body height,

speed, lateral drift, energy, and the operating range of both paradigms;

and (5) a reproducible finding that the model-based controller was coupled

to real time, whereas the learned policy was not.

Problem Statement and Research Questions

Given a single quadruped (Unitree Go2), a single simulator (Isaac Sim /

IsaacLab), and a single low-level joint-position interface, we address the

following questions:

- RQ1. How do a pre-trained RL policy and a hand-tuned IK/TROT

controller compare in speed, body-height stability, roll, lateral drift,

and cost of transport on flat terrain?

- RQ2. What is the operating range of each controller in commanded

speed?

- RQ3. Is the model-based controller's behavior reproducible, or is it

coupled to the simulator's real-time performance?

- RQ4. What are the practical integration defects that must be

addressed to run a model-based controller through a modern

GPU-accelerated simulator, and what is the limit of a simple

proportional attitude stabilizer?

Theory

This section presents the theory behind the implemented system. It first

describes the overall architecture and the two controllers, then summarizes

the practical integration problems and the telemetry used for the

evaluation. The emphasis is on the model-based controller, which is the

subject of this work, and on the points where it differs from the learned

policy.

System Architecture

The system consists of a Rust controller running in a container, a ROS 2

bridge, and the IsaacLab environment:

```mermaid

graph LR

RC["Rust controller<br/>(container, ROS 2)"] --> ROS["ROS 2 Jazzy<br/>/robot1/joint_group_controller/commands"]

ROS --> IS["IsaacLab 3.0<br/>ManagerBasedEnv"]

IS --> SIM["Isaac Sim 6.0<br/>PhysX (GPU)"]

SIM --> GO2["Unitree Go2"]

```

[image: figures/bw/fig_architecture.png]

Fig. 1. System architecture.

The controller publishes twelve joint-position targets as a

`std_msgs/Float64MultiArray`. IsaacLab applies them through position

actuators with a proportional–derivative law, emulating the low-level mode

of the real robot. The RL path uses the same asset and the same

joint-position interface; the policy is a TorchScript model executed inside

the simulator.

Robot and Actuation

The Unitree Go2 has twelve actuated joints: four legs (front-right,

front-left, rear-right, rear-left), each with a hip (ab/adduction), a

thigh, and a calf joint. Control is performed through target joint

positions and a PD law; the model-based controller uses stiffness 75 and

damping 0.5, while the NVIDIA policy uses stiffness 25 and damping 0.5, as

specified by its environment configuration.

Mathematical Formulation

The metrics and control relations used in this work are summarized below.

Stance foot velocity:

$$v_{\mathrm{st}} = -\frac{\mathrm{step_{dist}}}{4 dt \tau_{\mathrm{st}}} \tag{1}$$

where $\mathrm{step_{dist}}$ is the commanded stride distance, $dt$ is the

control time step, and $\tau_{\mathrm{st}}$ is the stance duration.

Joint torque estimate:

$$\tau_i = k_p (q_i^{\mathrm{cmd}} - q_i) - k_d \dot{q}_i \tag{2}$$

where $q_i^{\mathrm{cmd}}$ is the commanded joint angle, $q_i$ is the

measured angle, $\dot{q}_i$ is the joint velocity, and $k_p$ and $k_d$ are

the stiffness and damping gains.

Cost of transport:

$$\mathrm{CoT} = \frac{E}{m g d} \tag{3}$$

where $E$ is the mechanical energy, $m$ is the robot mass, $g$ is the

gravitational acceleration, and $d$ is the distance travelled.

Capture point:

$$y_{\mathrm{foot}} = y_{\mathrm{cm}} + \frac{v_y}{\omega} \tag{4}$$

$$\omega = \sqrt{\frac{g}{h}} \tag{5}$$

where $y_{\mathrm{foot}}$ is the required foot placement, $y_{\mathrm{cm}}$

is the center-of-mass position, $v_y$ is the lateral velocity, $\omega$ is

the natural frequency, $g$ is the gravitational acceleration, and $h$ is

the center-of-mass height.

Positive-feedback mode of the roll loop:

$$\dot{\varphi} = \alpha \varphi \tag{6}$$

where $\varphi$ is the body roll and $\alpha$ is the growth rate of the

unstable mode caused by the hip coupling.

Model-Based IK/TROT Controller

The controller is a finite-state machine with REST, STAND, and TROT

states. The TROT generator drives two diagonal pairs in antiphase (FR–RL

and FL–RR) with stance and swing phases and a double-support phase. Foot

trajectories are generated in the body frame: during stance the foot is

fixed in the world and therefore moves backwards in the body frame at the

commanded velocity; during swing a Raibert-style heuristic [8] places the

foot at the neutral point shifted by the velocity. The foot positions are

converted to joint angles by an analytic inverse-kinematics solution using

the Go2 link lengths (thigh and calf 0.213 m).

An attitude-compensation loop uses the IMU orientation to keep the feet

level: the pitch error is compensated by rotating the feet about the

lateral axis, and the roll error by a differential leg-length adjustment,

which avoids driving the hip joints into saturation. A proportional

yaw-stabilization term keeps the heading fixed.

Learned RL Policy

The RL baseline is the pre-trained NVIDIA policy distributed with Isaac

Sim [15]. Its observation is a 48-dimensional vector (base linear and

angular velocity in the body frame, gravity direction, commanded

velocities, joint position error from the default, joint velocities, and

the previous action). The policy outputs twelve actions at a decimated

rate, which are converted to joint-position targets. The policy is

executed step-by-step, so its behavior does not depend on wall-clock time.

Practical Problems and Their Solutions

Five concrete defects were found and fixed, each confirmed by telemetry

(125 columns: pose, angles, velocities, torques, foot positions and

contacts):

1. Stance foot drift. The stance foot velocity was computed as

$-\frac{\mathrm{step\_dist}}{4\,dt\,\tau_{\mathrm{st}}}$, where `stance_ticks` is the length

of one stance phase, whereas the leg is on the ground for several phases

in a row. The foot drifted backwards (up to −1.2 m) and the IK saturated

(`calf = 0`). It was replaced by the physically correct velocity,

Eq. (1) with v_st = -cmd_vel.

2. IMU compensation accumulation. The compensation was applied

incrementally to the gait state, so the tilt accumulated. It is now

applied only to the copy used for IK.

3. Inverted IMU sign. The compensation used `R(-comp)` instead of

`R(comp)`, amplifying the tilt. After correction, roll dropped from a

full flip (180°) to about 8°.

4. Inverted yaw sign. The yaw stabilization used `-0.5·yaw_err`, which

spun the robot up. With `+0.5·yaw_err` the heading is maintained.

5. Wall-clock PID. After switching the controller to a simulation-time

step, the PID still used wall-clock `dt`, causing a mismatch in the

integral and derivative terms.

A structural defect was also identified: compensating the roll by rotating

the feet created a positive feedback loop through the hip joints. The roll

is now compensated by a differential leg length, which does not actuate the

hip; the roll dropped from 50° to about 8° (Fig. 2).

[image: figures/bw/fig_roll.png]

Fig. 2. Body roll over time before (solid) and after (dashed) the differential leg-length fix.

Telemetry and Reproducibility

A unified telemetry system records 125 quantities per physics step in CSV

format: the body pose and orientation, linear and angular velocities in the

world and body frames, accelerations and the gravity direction, commanded

velocities and the controller mode, world positions and contacts of the

four feet, angles and velocities of the twelve joints, an estimate of the

joint torques, instantaneous powers and accumulated energy, joint tracking

errors, and flags for falls, hip saturation, and NaNs. The same format is

used for both controllers. The joint torque is estimated by Eq. (2); the energy is the integral of the

sum of joint powers, and the cost of transport is given by Eq. (3). To remove the

influence of performance, the model-based controller steps on simulation

time, published by the environment on a dedicated topic, which makes its

behavior deterministic and independent of the frame rate.

Experimental Results

This section reports the experimental comparison of the two controllers.

We first describe the setup and the metrics, then present the results of

forward walking, the operating range in commanded speed, additional

metrics, and the failure taxonomy with the negative results. All numbers

are reported as the mean and standard deviation over three runs unless

stated otherwise.

Setup

The simulator is Isaac Sim 6.0.1 with IsaacLab 3.0 on an Ubuntu 26.04

workstation with an NVIDIA RTX 5070 Ti GPU. The terrain is a flat plane

with a friction coefficient of 1.0. The robot is the Unitree Go2 with the

IsaacLab asset. The commanded velocity is 0.3 m/s (with a sweep from 0.1

to 0.4 m/s). Each controller was run three times; telemetry was recorded at

the physics rate.

Metrics

To compare the two controllers objectively we use a set of kinematic and

energetic metrics computed from the recorded telemetry. The distance

travelled and the mean speed characterize the locomotion performance; the

mean and standard deviation of the body height quantify how steadily the

robot holds its posture; the maximum roll and pitch quantify the body

attitude; the lateral drift quantifies how well the controller keeps a

straight line; the cost of transport (CoT) quantifies the energy

efficiency; and the time to reach a steady gait quantifies the transient

response. All metrics are averaged over the three runs of each controller.

Results

Table 1. Forward walking (vx = 0.3 m/s, mean ± std, n = 3).

| Metric | RL policy (NVIDIA) | IK/TROT (ours) |
| --- | --- | --- |
| Mean body height (m) | 0.159 ± 0.000 | 0.235 ± 0.001 |
| Std of height (m) | 0.040 ± 0.000 | 0.025 ± 0.001 |
| Mean speed (m/s) | 0.231 ± 0.000 | 0.215 ± 0.012 |
| Distance (m) | 3.34 ± 0.00 (15 s) | 2.75 ± 0.12 (19.3 s) |
| Lateral drift (m) | 0.37 ± 0.00 | 1.09 ± 0.16 |
| Max roll (deg) | 2.3 ± 0.0 | 27.8 ± 3.7 |
| Max pitch (deg) | 4.1 ± 0.0 | 35.6 ± 0.0 * |
| Falls | none | none |
| CoT | 2.84 | 2.83 ± 0.41 |

\* the IK pitch maximum is a startup transient; the steady-state pitch is

about 2°. The RL policy is deterministic across runs (standard deviation

zero). The trajectories of the two controllers are compared in Fig. 3.

[image: figures/bw/fig_trajectory.png]

Fig. 3. Trajectory in the horizontal plane: model-based IK/TROT (solid) and RL policy (dashed).

Table 2. Operating range (speed / roll / drift).

| vx (m/s) | RL policy | IK/TROT |
| --- | --- | --- |
| 0.1 | 0.026 / 2.6° / 0.09 m | 0.001 / 69° / 0.28 m |
| 0.2 | 0.116 / 3.6° / 0.21 m | 0.024 / 47° / 0.23 m |
| 0.3 | 0.222 / 2.3° / 0.37 m | 0.141 / 25° / 0.77 m |
| 0.4 | 0.344 / 3.0° / 0.80 m | 0.066 / 31° / 0.60 m |

The RL policy tracks the commanded velocity over the whole range with a

small roll, whereas the model-based controller is stable only near

0.3 m/s. The speed tracking of both controllers is shown in Fig. 4.

[image: figures/bw/fig_speed.png]

Fig. 4. Achieved speed versus commanded speed for the RL policy (circles) and the IK/TROT controller (squares); the dotted line is the ideal.

Additional Metrics

The body jerk (root-mean-square of the derivative of the body-frame

acceleration) is high for the model-based controller (about 370, 580, and

620 m/s³ on the three axes), reflecting the impulsive stance/swing

transitions. The energy distribution across joint groups is about 8% in

the hip joints, 37% in the thigh joints, and 55% in the calf joints; the

calf joints dominate, consistent with their role in supporting and

propelling the body.

Failure Taxonomy and Negative Results

Table 3. Failure modes of the model-based controller.

| Defect | Symptom | Root cause | Resolution |
| --- | --- | --- | --- |
| Stance foot drift | IK saturation (`calf=0`) | wrong velocity divisor | `velocity = -cmd_vel` |
| IMU accumulation | growing tilt | fed back into gait state | apply to IK copy only |
| Inverted IMU sign | flip over 180° | `R(-comp)` | correct sign, `kp=1.0` |
| Inverted yaw sign | yaw spin | `-0.5·yaw_err` | `+0.5·yaw_err` |
| Wall-clock PID | fps-dependent behavior | wall-clock `dt` | simulation-time PID |
| Roll via hip | hip saturated at −0.30 | positive feedback via hip | differential leg length |
| Forward-motion loss | lateral motion | wrong gait `time_step` | restore `time_step=0.02` |

Several plausible remedies did not help: increasing the attitude gain

(`kp=1.0→2.0`), relaxing the hip limit (`0.3→0.6`), inverting the

roll-compensation sign, and a symmetric stance/swing trot all degraded

stability. Only the differential leg-length compensation reduced the roll

substantially.

Conclusions

The two paradigms trade off differently. The learned policy is markedly

more stable, keeps the course, and works over a wide speed range; it is

the natural choice when a trained policy is available and a GPU is present.

The model-based controller is fully deterministic, transparent, requires no

GPU or training, and holds the body height more tightly, but it is stable

only in a narrow regime and has a residual roll and drift.

A key observation concerns time-base coupling. Before correction, the

model-based controller ran its 60 Hz loop on wall-clock time while the

simulation advanced by fixed steps; when the simulation slowed down, the

number of physics steps per control command changed, and the robot's

behavior changed with it. After switching the controller to simulation

time, the behavior became deterministic. The learned policy, being

step-based, never had this problem.

The residual roll of the model-based controller is not an implementation

defect but a reproducible limit of a simple proportional attitude

stabilizer. A linearized view explains why: compensating the roll by

rotating the feet necessarily actuates the hip, and if the sign of this

coupling reinforces the roll, the loop has positive feedback,

Eq. (6), limited only by the hip clamp. Compensating the roll through

the leg length removes the feedback and reduced the roll from 50° to about

8°. Full elimination requires placing the feet outside the center of mass —

a capture-point condition, Eq. (4) —

which the simple proportional loop does not satisfy.

The results are summarized as four propositions. **P1 (time-base

coupling):** a model-based controller that runs on wall-clock time is

unpredictably coupled to the simulator's real-time performance, whereas a

simulation-time loop is deterministic. **P2 (limit of a proportional

attitude stabilizer):** compensating roll by rotating the feet actuates

the hip and can create positive feedback; a differential leg length reduces

but does not eliminate it. P3 (stability versus determinism): the RL

policy is more stable and tracks a wider velocity range, while the

model-based controller is deterministic, training-free, and holds height

better; the two are complementary. P4 (comparable energy): despite the

stability gap, the two achieve a comparable cost of transport (2.83 vs

2.84).

Limitations. The study is simulation-only; sim-to-real was not

verified. The two controllers use different assets and PD gains, so part

of the difference may be attributable to these rather than to the control

paradigm. The model-based controller's operating range is narrow. The RL

policy is a ready-made NVIDIA model and was not trained by us.

Future work. Implement a capture-point balancer and a turning

controller to eliminate the residual roll and drift; equalize the

comparison conditions by running both controllers on a single asset with

identical PD gains; conduct disturbance experiments (lateral push,

slippery surface); and transfer the result to a real robot.

Acknowledgment

The author thanks the Department of Robotics and Mechatronics of Bauman

Moscow State Technical University for supporting this work.

References

[1] Unitree Robotics, "Unitree Go2 — quadruped robot and SDK," documentation, 2024.

[2] NVIDIA, "Isaac Lab: A unified and modular framework for robot learning," documentation, 2024.

[3] NVIDIA, "Isaac Sim," documentation, 2026.

[4] J. Hwangbo et al., "Learning agile and dynamic motor skills for legged robots," *Science Robotics*, vol. 4, no. 26, 2019.

[5] J. Lee, J. Hwangbo, L. Sentis, V. Kim, and P. Fankhauser, "Learning quadrupedal locomotion over challenging terrain," *Science Robotics*, vol. 5, no. 47, 2020.

[6] T. Miki et al., "Learning robust perceptive locomotion for quadrupedal robots in the wild," *Science Robotics*, vol. 7, no. 62, 2022.

[7] N. Rudin, D. Hoeller, P. Reist, and M. Hutter, "Learning to walk in minutes using massively parallel deep reinforcement learning," in *Proc. Conf. Robot Learning (CoRL)*, 2021.

[8] M. H. Raibert, *Legged Robots That Balance*. Cambridge, MA, USA: MIT Press, 1986.

[9] B. Katz, J. Di Carlo, and S. Kim, "Mini Cheetah: A platform for pushing the limits of dynamic quadruped control," in *Proc. IEEE Int. Conf. Robotics and Automation (ICRA)*, 2019, pp. 6295–6301.

[10] J. M. Jimeno, "CHAMP: Controller for highly agile multi-legged platforms," GitHub repository, 2021.

[11] G. Bledt et al., "MIT Cheetah 3: Design and control of a robust, dynamic quadruped robot," in *Proc. IEEE/RSJ Int. Conf. Intelligent Robots and Systems (IROS)*, 2018, pp. 2245–2252.

[12] *Benchmarking MPC and RL for legged robot locomotion in MuJoCo*, arXiv:2501.16590, 2025.

[13] *Isaac Sim-to-real: RL-based locomotion for quadrupeds*, arXiv:2607.18135, 2026.

[14] *Kine2Go: A kinematic dataset for the Unitree Go2*, arXiv:2606.14433, 2026.

[15] NVIDIA, "Isaac Gym: High performance GPU-based physics simulation for robot learning," arXiv:2108.10470, 2021.